| SoftAP Mode - features | 8188cus/8192cu/8192DU-VS |
| --- | --- |
| Hosted Daemon | HOSTAPD + Realtek proprietary driver interface for 871x-based |
| Support MAX. number of stations | 8 Clients |
| Security Mode: - Open - WEP Open/Shared - TKIP - CCMP/AES | - Yes - No. - No. - Yes |
| Authentication methods: - WPA-PSK ("WPA-Personal") - WPA with EAP (e.g., with RADIUS authentication server) ("WPA-Enterprise") | - Yes. - No. |
| Wireless Mode - B/G/N/A | Yes (A Band for 8192DU-VS only) |
| a. fragmentation b. defragmentation | a. No b. Yes |
| 802.11N HT – Band Width 20/40 MHZ | yes |
| 802.11N HT - AMPDU TX-AMPDU RX-AMPDU | Tx: yes Rx: yes |
| 802.11N HT - AMSDU TX-AMSDU RX-AMSDU | Tx: No. Rx: yes |
| Intra-BSS distribution | yes |
| legacy power saving | yes |
| WPS v1.1 - internal registrar | yes, PIN/PBC Methods |
| 802.11h for 5GHz | No. |